DigiMemo Ink File Format

Version:	1.00

Date: 2004.9.9

Decsription:

This document describes the file format for the Ink file (.DHW file) which generated by the DigiMemo. Each Ink file corresponding to a single page that user assigned to store the handwriting notes on the DigiMemo.

There are two fileds in the Ink file: File Head and Stroke data. The File Head contains the ID string,

version number and page information. The Stroke data is formed by the following data type: Layer data, Timestamp data, Pen state data and Coordinate data. Please note that not all data types will be appeared in every Stroke data, it will depends on the situations when the Stroke data is generated.

Definition and Format:

1. File Head

| Name | Length | Locate | Content |
| --- | --- | --- | --- |
| ID string | 32 | 00h ~ 1Fh | ACECAD_DIGIMEMO_HANDWRITING_____ |
| Version | 1 | 20h | 01h |
| Page size | 4 | 21h ~ 24h | 38h, 17h, 10h, 21h (Width: 1738h, Height:2110h) |
| Page type | 1 | 25h | 00h (0-A5, 1-A4, … , 8-B5, 9-B4) |
| (retain) | 2 | 26h ~ 27h | Fill with 00h |

2. Stroke Data

2-1. Layer Data Format (2 bytes)

Storage	|	MSB							LSB	|

Sequence	|	7	6	5	4	3	2	1	0	|

Byte1		|	1	0	0	1	0	0	0	0	|	layer data type (90h)

Byte2		|	0	L6	L5	L4	L3	L2	L1	L0	|	layer number (1 ~ 7Fh)

The Layer data is appended to the tail of the Ink file every time the Ink file was closed. The Ink file will be closed at anyone of the following situations occurred:

a. The user switch to another page.

b. The user manually power off the DigiMemo.

c. The DigiMemo enter the sleep mode.

d. The battery cover was opened.

2-2. Timestamp Data Format (2 bytes) ** Reserved **

Storage	|	MSB							LSB	|

Sequence	|	7	6	5	4	3	2	1	0	|

Byte1		|	1	0	0	0	1	0	0	0	|	Timestamp data type (88h)

Byte2		|	0	T6	T5	T4	T3	T2	T1	T0	|	Timestamp number (1 ~ 7Fh)

Note: base on 20ms

Note: This function was disabled in this version of the file format. The Timestamp number will always be filled with 7Fh.

The Timestamp data is added to the beginning of every stroke. It indicates the time period between the current (following) stroke and last stroke. The unit of the timestamp number is 20ms. For example, if you start the new stroke within 20ms after the last stroke, the Timestame number will be 0. If you start the new stroke after 45ms from the end of the last stroke, then the Timestamp number will be 2. This is for the recognition software to simulate the real time handwriting.

2-2 Pen State Data Format (1 bytes)

Storage	|	MSB							LSB	|

Sequence	|	7	6	5	4	3	2	1	0	|

Byte1		|	1	0	0	0	0	C1	C0	B	|

B: 0 = Pen up, 1 = Pen down

C0 ~ C1  Pen Color : 00 = Black (default) , 01 = Red , 10 = Blue , 11 = Green

The Pen state data include the pen tip status and pen ink color information. Currently, the ink color is fixed to Black. The pen tip status data indicates the beginning and ending of a single stroke (Please refer to the Ink file sample below).

2-3 Coordinate Data Format (4 bytes)

Storage	|	MSB							LSB	|

Sequence	|	7	6	5	4	3	2	1	0	|

Byte1		|	0	X6	X5	X4	X3	X2	X1	X0	|

Byte2		|	0	X13	X12	X11	X10	X9	X8	X7	|

Byte3		|	0	Y6	Y5	Y4	Y3	Y2	Y1	Y0	|

Byte4		|	0	Y13	Y12	Y11	Y10	Y9	Y8	Y7	|

X0 ~ X14: X axis coordinate value

Y0 ~ Y14: Y axis coordinate value

The Coordinate data indicates the position of a single point within the stroke. The data is in the absolute coordinate value to indicate the horizontal and vertical distance between the point and the origin. The origin is inthe lower-left corner of the paper.

Ink File Sample

[File Header]

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]	<-- 1st stroke

[Timestamp]		<-- Beginning of 2nd stroke

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

[Timestamp]		<-- Beginning of 3rd stroke

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

.

.

.

[Timestamp]		<-- Beginning of n stroke

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

[Layer 0]			<-- First layer end here

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

[Timestamp]

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

.

.

[Timestamp]		<-- Beginning of n stroke

[Pen Down][X1,X2,Y1,Y2][[X1,X2,Y1,Y2]....[X1,X2,Y1,Y2][Pen Up][X1,X2.Y1.Y2]

[Layer 1]			<-- Second layer end here

Note:

1.	There is no Timestamp data before the first stroke of each layer.

2.	The coordinate data after [Pen Up] indicate the position that the pen is released.